# UFGS 05 30 00 – References to Remove (Material-Only Scope)

Purpose: This document highlights references and clauses in UFGS 05 30 00 (Steel Decks) that should be REMOVED when the section is limited strictly to MATERIAL SPECIFICATIONS and excludes design criteria and insurance-based requirements.

## REMOVE – Design Criteria References

UFC 3-301-01 – Structural Engineering

International Building Code (IBC)

SDI DDM04 – Diaphragm Design Manual

ICC-ES AC43 / Evaluation Service Reports

AISC 360 references related to shear connector design

AISI D100 references tied to diaphragm or structural performance

Engineering responsibility or sealed calculation requirements

Deck diaphragm or lateral-force-resisting language

## REMOVE – Insurance / Performance-Based References

Factory Mutual (FM) Approval Guide

FM Data Sheets 1-28 and 1-28R

FM uplift pressure or safety factor requirements

UL 580 uplift resistance testing

UL Fire Resistance Directory

UL classified assemblies

NFPA 70 (unless deck used as raceway)

UL 209 (unless cellular deck used as raceway)

## KEEP – Material Specifications (Do Not Remove)

ASTM steel material standards (A653, A792, A1008, etc.)

SDI C / NC / RD for section geometry

Coating and galvanization requirements

Fastener material specifications (non-FM/UL)

Fabrication tolerances

Accessories and closures

## Summary Rule

If a reference governs loads, diaphragm behavior, structural performance, or insurance compliance → REMOVE.
If a reference governs material composition, coating, or geometry → KEEP.